Dwell Calculation

Current State & Proposed Improvements

March 2026

# How It Works Today

The system checks every timepoint: did the bus leave more than 3 minutes late?

- No — no dwell recorded

- Yes, but arrived on time — dwell = how late it departed

- Yes, and arrived late — dwell = time sitting at the stop

March 3 Baseline:

| Metric | Value | Threshold |
| --- | --- | --- |
| Total dwell hours | 14.0 | All dwell > 0 |
| Moderate incidents | 122 | 2–5 minutes |
| High incidents | 19 | > 5 minutes |

# What We Want to Change

## 1. Subtract Normal Boarding Time (90 seconds)

Right now, routine passenger boarding counts as dwell. A bus that stops for 60 seconds to pick up riders looks the same as one stuck at a signal for 60 seconds.

Change: Subtract 90 seconds of normal boarding before counting dwell.

Impact: March 3 total dwell drops from ~14 hrs to ~10–11 hrs.

## 2. Lower the Late Gate from 3 Minutes to 2 Minutes

A bus departing 2:59 late records zero dwell. One departing 3:01 late records full dwell. We're missing real delays in that gap.

Change: Start counting dwell at 2 minutes late instead of 3.

Impact: Captures more incidents, adding ~2–3 hrs before the boarding buffer is applied.

## 3. Fairer Terminal Dwell

At terminals, buses have scheduled recovery time (5–15 min). If a bus arrives early and uses most of that recovery but still leaves a few minutes late, the current formula counts all the lateness as dwell — even though the bus barely exceeded its scheduled time.

Change: Cap dwell at the actual extra time beyond scheduled recovery, not total departure lateness.

Impact: Reduces inflated dwell at terminal stops.

# Combined Effect (March 3 Estimate)

| Scenario | Total Hours | Moderate | High |
| --- | --- | --- | --- |
| Current | 14.0 | 122 | 19 |
| With all three changes | ~8–10 | ~90–100 | ~15–18 |

Lower total hours because we remove boarding noise. Slightly fewer incidents because borderline cases get filtered. The remaining incidents are more meaningful — real delays, not routine operations.

# Implementation Order

- Boarding buffer — biggest impact, simplest change

- Lower gate — one constant change

- Terminal cap — slightly more involved but straightforward

Each change can be applied and tested independently.